ELECTRICAL AND ELECTRONIC TECHNOLOGY

Time: 40 mins

Time: 40 mins

RCL PROBLEM SOLVING EXERCISES

NOTE: This worksheet follows from the power-point lesson on RCL Circuits. It is 	     	        recommended that the following problem solving questions be attempted.

Instruction: The questions below involves problem solving exercises on RCL circuits. 	  		   The estimated time for the completion of question 1 is 10minutes. Question 			    2 and 3 are allocated an estimated time of 15 minutes each.

- At a given frequency, a series AC circuit has an inductive reactance (XL) of 40Ω, a capacitive reactance (XC) of 80Ω and a resistance (R) of 30Ω. Calculate the

- total impedance (ZT) of the circuit

- magnitude of the phase angle of the circuit.      (CXC EET 2019)

R

110 V

XL=180Ω

XC=150Ω

R = 50Ω

FIG 1.

R

110 V

XL=180Ω

XC=150Ω

R = 50Ω

FIG 1.

For the circuit shown in fig 1 above calculate

- Total impedance ZT

- Total current IT

- Voltage across R, XL, XC

- Power factor

- True power utilized by the circuit

R

R = 50Ω

XL=0.199H

XC= 26.52µf

240 Vac , 60HZ

Fig 2

R

R = 50Ω

XL=0.199H

XC= 26.52µf

240 Vac , 60HZ

Fig 2

For the circuit above in Fig 2., calculate the

- total impedance, ZT

- total current, IT

- voltage across

- R

- XL

- XC

- power factor

- circuit True power in watts

RCL CIRCUITS SOLUTIONS

1. (a) ZT =    R2 + (XC – XL)2       ;  R = 30Ω; XL = 40Ω; XC = 80Ω

ZT =    302 + (80 – 40)2                =     302 + (40)2           =   900 + 1600

ZT = 2500          = 50Ω

- Phase angle of the circuit, cos ø = R/Z = 30/50 = 0.6

Therefore; ø = cos-1 0.6 = 53.1°

R

110 V

XL=180Ω

XC=150Ω

R = 50Ω

FIG 1.

R

110 V

XL=180Ω

XC=150Ω

R = 50Ω

FIG 1.

2.

2. (a)

ZT =    R2 + (XL - XC)2       ;  R = 50Ω; XL = 180Ω; XC = 150Ω

ZT =    502 + (180 – 150)2                =     502 + (30)2           =   2500 + 900

ZT = 3400          = 58Ω

- IT = V/R = 110/58 = 1.9A

2. (c) (i) VR = I x R = 1.9 x 50 = 95 V

(ii) VL = I x XL = 1.9 x 180 = 342V

- VC = I x Xc = 1.9 x 150 = 285 V

(d) Power Factor (Cos ø) = R/Z = 50/58 = 0.86

(e) P = VxI x cos ø = 110 x 1.9 x 0.86 = 180W

OR

P = I2 x R = (1.9)2 x 50 = 180W

R

R = 50Ω

XL=0.199H

XC= 26.52µf

240 Vac , 60HZ

Fig 2

R

R = 50Ω

XL=0.199H

XC= 26.52µf

240 Vac , 60HZ

Fig 2

3.

- XL = 2ПfL = 2 x 3.142 x 60 x 0.199 = 75Ω

XC = 1/2ПFC = 1/ 2 x 3.142 x 60 x 26.52 x10-6 = 100Ω

R = 50Ω(a)

ZT =    R2 + (XC – XL)2       ;  R = 50Ω; XL = 75Ω; XC = 100Ω

ZT =    502 + (100 – 75)2                =     502 + (25)2           =   2500 + 625

ZT = 3125          = 55.9Ω

- IT = V/R = 240/55.9 = 4.29A

(c)(i) VR = I x R = 4.29 x 50 = 214.5 V

(ii) VL = I x XL = 4.29 x 75 = 321.75V

(iii)VC = I x Xc = 4.29 x 100 = 429 V

(d)Power Factor (Cos ø) = R/Z = 50/55.9 = 0.894

(e) VxI x cos ø = 240 x 4.29 x 0.894 = 920W

OR

P = I2 x R = (4.29)2 x 50 = 920W