ELECTRICAL AND ELECTRONIC TECHNOLOGY

RESISTIVITY PROBLEM SOLVING EXERCISES

{R = ρ L}

A

NOTE: This worksheet follows from the power-point lesson on Physical factors that

affects the resistance of a conductor. It is recommended that the following

problem solving questions be attempted.

- Calculate the length of copper wire, 1.5mm diameter, to have a resistance of 0.3Ω, given that the resistivity of the copper is 0.017µΩm.

- The resistance of 7.7m of manganin wire, of uniform cross-section, 1.3mm2, is 2.6 Ω. Calculate the resistivity of manganin in ohmmeters.

- In a test on a 100mm strip of copper, the resistance was found to be 171µΩ. The cross-sectional area is 9.92mm2. Calculate the resistivity of the copper.

- A heating coil of power rating 10W is required when the p.d. across it is 20 V. Calculate the length of nichrome wire needed to make the coil of cross-sectional area of the wire used is 1 x 10-7m2 and the resistivity of nichrome is 1 x 10-6 Ωm.

SOLUTION

- Calculate the length of copper wire, 1.5mm diameter, to have a resistance of 0.3Ω, given that the resistivity of the copper is 0.017µΩm.

Outline Info:

R = 0.3Ω 						L =?

ρ = 0.017 µΩm = .017 X 10-6 Ωm			A =?

- since, d = 1.5mm = 1.5 x 10-3 m, we can find A

- A= πd2       =   3.14 x (1.5 x 10-3)2       = 3.14 x 2.25 10-6        = 7.065 x 10-6

4		        4	    4			     4

A = 1.77 x 10-6 m2

To find length, L

L =  RA         =       0.3 x 1.77 x 10-6	         0.3 x 1.77 x 10-6	       	    0.531

Ρ                          .017 X 10-6           =         .017 X 10-6                     =	     0.017   = 31.2m

Therefore, length, L = 31.2m

SOLUTION

- The resistance of 7.7m of manganin wire, of uniform cross-section, 1.3mm2, is 2.6 Ω. Calculate the resistivity of manganin in ohmmeters

Outline Info:

R = 2.6Ω 						L = 7.7 m

ρ =?			                                                     A = 1.3mm2 = 1.3 x (10-3)2m

Hence, A = 1.3 X 10-6 m

ρ = RA

L

ρ =  2.6 X  1.3 X 10-6              = 3.38 X  10-6                =     0.44 X 10-6 Ωm    =  0.44 µΩm

7.7	7.7

Therefore , Resistivity, ρ = 0.44 µΩm

SOLUTION

- In a test on a 100mm strip of copper, the resistance was found to be 171µΩ. The cross-sectional area is 9.92mm2. Calculate the resistivity of the copper.

Outline Info:

R = 171µΩ = 171 x 10-6Ω				L = 100mm = 100 x 10-3m

ρ =?			                                                     A = 9.92mm2 = 9.92 x (10-3)2m

Hence, A = 9.92 X 10-6 m

ρ = RA

L

ρ = 171 x 10-6X 9.92 X 10-6     = 1696 X 10-12        =   16.96 X 10-12 =16.96 X 10(-12- -3) = 16.96 x 10-9 Ωm

100 x 10-3	   100 x 10-3                   10-3

Therefore, Resistivity, ρ = 16.96 x 10-9 Ωm

SOLUTION

- A heating coil of power rating 10W is required when the p.d. across it is 20 V. Calculate the length of nichrome wire needed to make the coil of cross-sectional area of the wire used is 1 x 10-7m2 and the resistivity of nichrome is 1 x 10-6 Ωm.

Outline Info:

P = 10 W; V = 20V; hence to find R;

P = V2/R:  R = V2/P   = (20)2/10 = 400/10 = 40Ω

Now:

ρ = 1 x 10-6 Ωm:         A = 1 x 10-7m2   and R = 40 Ω

To find length, L

L =  RA         =       40 x 1.0 x 10-7	         40 x 1.0 x 10-7	       	40 x 10-1

Ρ                          1.0 X 10-6           =         1.0 X 10-6                     =	     1              = 4m

Therefore, length, L = 4 m